EXP-1 1.1

clc;

clear

Fs = 10000;

Fp = 1000;

Fst = 1500;

% Minimum order calculation

[N,Wn] = buttord(Fp/(Fs/2), Fst/(Fs/2), 1, 60);

[b,a] = butter(N,Wn);

% Magnitude & Phase

figure

freqz(b,a);

title('Magnitude & Phase Response');

% Impulse Response

figure

[h,n] = impz(b,a,50);

stem(n,h);

title('Impulse Response');

% Step Response

figure

stepz(b,a);

title('Step Response');

% Pole-Zero Plot

figure

zplane(b,a);

title('Pole-Zero Plot');

% Print results

fprintf('Minimum Filter Order (N) = %d\n',N);

fprintf('Cutoff Frequency (Wn) = %.4f\n',Wn);

EXP-1 (1.2 )

clc; clear;

% EX NO:1

% Butterworth IIR Filters

% DATE:

% 1.2. Design and Analysis of a Butterworth IIR High-Pass Filter Using MATLAB

Fs = 10000; % Sampling frequency

Fp = 3000; % Passband frequency

Fst = 2000; % Stopband frequency

% Butterworth filter order and cutoff

[N, Wn] = buttord(Fp/(Fs/2), Fst/(Fs/2), 1, 60);

[b, a] = butter(N, Wn, 'high');

% Magnitude & Phase Response

figure;

freqz(b, a);

title('Magnitude and Phase Response');

% Impulse Response

figure;

[h, n] = impz(b, a, 50);

stem(n, h, 'filled'); grid on;

title('Impulse Response');

% Step Response

figure;

stepz(b, a);

title('Step Response');

% Group Delay

figure;

grpdelay(b, a);

title('Group Delay');

% Pole-Zero Plot

figure;

zplane(b, a);

title('Pole-Zero Plot');

% Display results

fprintf('Minimum Filter Order (N) = %d\n', N);

fprintf('Cutoff Frequency (Wn) = %.4f\n', Wn